Technical Specifications

RET Control Cable din 8pin M/F

# ETL-AISG-Din Male to Female

# Main Features

AISG 3.0 Compliant RET Control Cable Feeds Data & DC Power to RET Components

# Electrical Data

| Protocol | 3GPP/AISG 3.0/AISG 2.0/AISG 1.1 |
| --- | --- |
| Voltage, Rated | 300 V |

Environmental Data

Operating Temperature Range	-40°C to +70°C | -40°F to +158°F Relative Humidity	5% - 95%

Weather Protection	IP67

# Mechanical Data

AISG Connector A	8-Pin DIN Female, gold plated

AISG Connector B	8-Pin DIN Male, gold plated

Braid Coverage	Coverage Rate ≥85 %

Data Conductor Type	0.32 mm² (22 AWG) twisted pair

Data Conductors, Quantity	2

Power Conductor Type	0.75 mm² (19 AWG) stranded

Power Conductors, Quantity	4

Total Conductors, Quantity	6

Shield Material	Aluminum

Jacket Material	UV protection ( Made in Malaysia)

Color	Black

Diameter Over Jacket	8.00 mm